This Report must not be construed as altering the contract and is issued solely to draw to your attention any variances from the Contract, requirements observed during our construction review, or, to interpret a Contract requirement.  Implementation of any item which may alter the Contract shall not commence until authorized by Change Order.  Review has been carried out by examination of selected samples of the Work shown on Contract documents prepared by this firm.  Responsibility for compliance with the plans and specifications rests with the Contractor.

- INTRODUCTION

The Geotechnical investigation of the abovementioned property was completed on the 27th of August 2026, at the request of the client, Britt Bagnall, c/o Oasis Pools. The purpose of this report is to provide a detailed outline of the subsurface conditions encountered beneath the site to aid in the structural design of the proposed pool and cabana.

The inspection was carried out by Mr. Alexander Moroney, MSc., of AMR Consulting Engineers.

  - Site Work

- Two (2 No.) trial pits using a backhoe supplied by Slims Heavy Equipment Services Ltd., to assess the in-situ subsurface conditions. The soil descriptions below are based on median soil gradation and consistencies encountered during the investigation.

- RESULTS

  - Trial Pit Results

Depth of Trial Pits:

| Trial Pit No. | Groundwater Recharge Level | Final Depth of Pit | Competent Stratum Encountered |
| --- | --- | --- | --- |
| 1 | 6’-3” | 6’-4” | Ironshore Formation Limestone |
| 2 | 6’-0” | 6’-4” | Ironshore Formation Limestone |

- DISCUSSION

Subsurface Commentary – Trial Pits

Native Bedrock was not encountered within any trial pits. Dense Ironshore Formation Limestone was encountered across the site at 6’-4” below grade level. The Ironshore Formation is the relic of a back reef lagoon palaeo-setting within the Pleistocene epoch, a low-energy environment where fine carbonate sediments would have accumulated between high-standing Ironshore patch reefs, situated further inland during a time of higher sea level, and eventually becoming the foundation for mangrove swamps due to the gradual fall in sea level. Consolidated marl typically has an allowable bearing capacity of approximately 3,000 psf.

    - Soil Descriptions

Strata: MADE GROUND, ORGANIC TOPSOIL

Trial pit No. encountered: All Pits

Thickness range: 0’-9’’ to 0’-10”

Underlying: On grade

- Description: Loose brown, dark-brown, organic topsoil. Loose, compressible organic silt (≤0.002”) soil with inclusions of vegetation and roots. Uniformly graded compressible organic silt is not suitable for bearing, and not suitable for use within the ground-filling profile. Suitable for agricultural and landscaping use.

Strata: MADE GROUND, MARL FILL

Trial pit No. encountered: All Pits

Thickness range: 2’-6” to 3’-11”

Underlying: Organic Topsoil

- Description: consolidated, off-white, gravel soil. Predominantly angular, medium-sized gravels (0.24” through to 0.8” dia.). Secondary constituents of fine sand (0.008” through to 0.024” dia.). Well-graded soil suitable for reuse within the ground-filling profile, not suitable for immediate bearing/contact beneath foundation systems.

NOTES:

All TPs:

- Excavation carried out successfully

- SITE CLASSIFICATION FOR SEISMIC DESIGN

In the absence of seismic survey data, the default seismic site class is D. It is the opinion of this practice that the site class will be unlikely to change following a seismic survey given the current quantity and thickness of the sand layer across the site. However, ground remediation works, such as replacing the sand with engineered fill, is likely to increase the grade at least to site class C.

- CONCLUSION

From the results of the ground investigation, the subsurface conditions across the site consist of clean marl. Given the structural loadings of the proposed pool and cabana it is the opinion of this practice that the underlying Ironshore Formation limestone may be relied upon as a bearing stratum. The recommended foundation is as follows:

  - Recommended Foundation – Ground Bearing Slab (pool) and Strip Footings (Cabana)

A viable foundation choice based on the presence of shallow, dense Ironshore Formation limestone across the site is the use of a ground bearing slab. This foundation system would involve the construction of a monolithic slab foundation reinforced with rebar at the required depth below grade. The slab should include integrated perimeter edge thickeners to provide adequate space for the required rebar at the slab/wall interface for the pool; additional rebar will likely be required to account for the concentrated loads around the edges of the pool. The foundation will bear on clean underlying native marl beneath the proposed final depth of the pool.

Additionally, we propose the use of isolated pad footings and linear strip footings, to supporting the cabana. The ground floor slab will likely be designed as cast on grade. It is therefore recommended that the crusher run fill for the foundation system should bear on the clean marl fill encountered between 0’-9” to 0’-10” below grade level.

Any contaminated and or compressible soils will need to be demucked and removed prior to depositing clean engineered fill beneath the footprint of the pool. Any other contaminated and compressible soils encountered throughout future earth works should be excavated and replaced with suitable engineered fill. After the placement of imported engineered fill, additional and thorough compaction is recommended across the site followed by a modified proctor test to ensure compaction has been achieved (via. TBM Engineering Ltd.).

  - It is important to note, when building a foundation system on engineered fill there is the possibility of differential settlement of the foundation system (this could lead to cracking in walls and slabs). If this method is to be used, the following technical requirements must be strictly adhered to:

    - Well-constructed excavation, safely executed, with all soft and hard spots removed, and made reasonably dry and well drained.

    - Sound fill without undesirable material and capable of compaction as specified, provided with starting and capping layers as necessary.

    - Placement and compaction to ensure that the performance of the fill will meet required criteria as a foundation fill (Typically 95% compaction or greater in layers no greater than 9”). Fill material specification to be generated and approved by a professional engineer.

    - Appropriate monitoring and quality control throughout development.

See Appendix A for Site Figures
See Appendix B for Photo Gallery
See Appendix C for Trial Pit Logs
See Appendix D for Limitations

I trust this information is satisfactory and if you have any questions regarding the above or enclosed, please don’t hesitate to contact me.

Yours faithfully,

M. Alexander Moroney, Msc

Engineering Geologist/ Geotechnical Engineer

For and on behalf of

AMR Consulting Engineers.